Multirotor Flight Testing: Clarification for DARPA LIFT Challenge

How Test Flights Support Our Primary UAS Design

| Submitted by | Natan Vidra & Team Anote |
| --- | --- |
| Organization | Anote, Inc. |
| Date | June 11, 2026 |
| Re | DARPA LIFT Challenge — Response to Reviewer Query |

Executive Summary

The multirotor flight tests visible in our uploaded footage are physics validation and calibration flights, not demonstrations of our final competition design. These flights are a deliberate step in our AI-driven design pipeline: we use real flight data to ground-truth the physics models that our optimizer relies on before committing designs to hardware. Our primary UAS design concept is detailed in the attached Concept Paper (Anote_Final_Concept_Paper.pdf).

How Multirotor Testing Fits the Design Pipeline

Stage 1 — Physics Model Calibration (Multirotor Test Flights)

We fly off-the-shelf multirotor platforms (quadcopters and hexacopters) to validate the core physics relationships our optimizer uses. The test vehicles are not the competition design — they are instrumented benches used to confirm that our simplified actuator disk model, structural equations, and feasibility constraints reflect real-world behavior before we run the optimizer at scale.

| Model Parameter | Formula | Validated Against |
| --- | --- | --- |
| Thrust | T = 10 × N_motors × D_prop² × throttle | Hover thrust at varying throttle |
| Payload capacity | payload = (T / 9.81) − total_mass | Known payload added incrementally |
| Structural safety factor | mean_UTS / 100 MPa | Load deflection of CF vs. Al frames |
| Thrust-to-weight ratio | T / (mass × 9.81) ≥ 1.5 | Minimum throttle at various mass configs |

Stage 2 — AI-Driven Design Optimization

With validated physics models, we run our optimization pipeline (src/darpalyft/) over a large design space. The optimizer maximizes score = payload_kg / total_mass_kg subject to all physics and structural constraints, using Pareto front analysis to surface non-dominated designs.

Materials: carbon fiber, aluminum alloy, titanium, fiberglass, ABS plastic

Motor count: 4, 6, or 8

Propeller diameter: 0.2 – 0.5 m

Battery capacity: 400 – 1,200 Wh

Component masses: per-component continuous variables

Stage 3 — Primary Design Selection & Refinement

The highest-scoring feasible designs from Stage 2 feed into our primary UAS design (detailed in the Concept Paper). The Concept Paper describes the specific airframe configuration, propulsion system, and structural approach we are proposing for the competition — informed by, but distinct from, the generic multirotor test platforms.

Why Test Multirotors Instead of the Final Design Directly?

Testing the final design first would conflate calibration errors with design errors. By validating physics on known, off-the-shelf platforms — where ground truth is well-characterized — we ensure our optimizer is working from accurate physical models. Only after that validation do we trust the optimizer's output as a guide for the competition design. This approach also lets us iterate quickly: multirotor test platforms are cheap and fast to reconfigure, while the primary design involves more specialized manufacturing.

Reference Documents

| Document | Location | Description |
| --- | --- | --- |
| Concept Paper | docs/Anote_Final_Concept_Paper.pdf | Full primary UAS design concept for DARPA LIFT |
| Physics model | src/darpalyft/core.py | Thrust, structural, and payload calculations |
| Optimizer | src/darpalyft/core.py — DesignOptimizer | Design search and feasibility filtering |
| Evaluation metrics | src/darpalyft/evaluate.py | Payload-per-weight scoring and Pareto analysis |
| Optimization pipeline | scripts/run_optimization.py | End-to-end run script |

Contact

Natan Vidra — natan@anote.ai  |  Anote, Inc. — anote.ai